### SECTION 1 : GENERAL SAFETY AND HEALTH PROCEDURES

# PREVENTION OF ACCIDENTS

Every Contractor worker has the responsibility to act positively to prevent accidents.

#### HOUSEKEEPING

Good housekeeping at all work areas is essential to accident, injury and fire protection

It is the responsibility of every individual to:-

- Keep benches, table and floors clear of all material and equipment not required.

- Store inflammable materials to be stored in a safe area.

c)	Store wire, ropes, chains and hoses to be stored at appropriate place to avoid tripping 	hazards.

# ALCOHOL AND DRUGS

Alcoholic drinks are strictly forbidden on site.

Disciplinary actions will be taken against employees under influence of alcohol or drugs.

# EMERGENCY PROCEDURES / DRILLS

All personnel responsible for the operations of Emergency Procedures must familiarize themselves with their duties and carry them out promptly and efficiently.

# GENERAL DUTIES OF AL PERSONNEL

Safe practices and precautions are the responsibility or everyone, and all personnel must be aware of the safe codes of practice applicable to their trade.

All task must be performed in accordance with the safety rules outlined in this manual.

Always be alert to safety and health hazards. Where feasible, correct or eliminate an obvious hazard yourself. Report all hazard, correctives measures taken, of the absence or in adequacy of precautions in connection with this hazards to your  Safety and Health Officer.

Where any means or appliance is provided to secure health or safety, personnel must make of such items e.g safety spectacles, goggles, protective clothing, breathing apparatus, guard of machinery, etc

# PERSONAL PROTECTIVE EQUIPMENT

All personnel will be suitable equipped with personals protective equipment and additional equipment  dictated by the task.

Protective equipment must be used when the job or  the conditions warrant its use.

# EMPLOYMENT OF QUALIFIED SAFETY AND HEALTH OFFICER

The officer who is responsible for an operation or work site must:-

- Fully familiarize himself with the Contractors Construction Safety and Health manual.

- Ensure that personnel in his team are adequately trained in the use of all equipment and are fully aware of any potential hazards peculiar to their work.

- Ensure that the persons under his control clearly understand the safe practices and conditions of the job which they are about to undertake.

- Set good examples by adopting safe behavioral and promoting safe and health environmentally practices.

- Ensure that all employees in his team know what to do in the case of fire and know the location of and how to use fire fighting equipment.

- Ensures that adequate supervision is available at all times.

- Ensure that all machinery and equipment are properly maintained and inspected. Maintain good housekeeping within their teams all times.

- Investigate accidents, fires and dangerous occurrences and take effective remedial action to prevent recurrence.

- Ensure that personnel are acquaintance with the Emergency Procedures and conduct regular drill / exercises.

- Be familiar with and observe all regulation applicable to current operations and working all opportunities available for safety training

# FORMING OF SAFETY AND HEALTH COMMITTEE

A Safety and Health Committee is to be established for every job site consisting of Contractors Project Manager, Contractor Safety and Health Officer & The Construction Superintendents. The objectives and responsibilities of the Safety and Health Committee are:-

- To take all possible measures to promote Safety and Health awareness and prevent accidents by stressing the need to comply with all safety rules, procedure and policies.

- To encourage individuals to identify safety hazards and eliminate them or refer them to their respective supervisor  for action.

- To improve safety and accident prevention to the highest possible level. Officer responsible for the operation shall organize and chair the safety meeting which is to be held weekly on site immediately after a safety patrol is made.

All personnel working for the particular operations shall attend this Safety and Health meeting. During the Safety and Health meetings, the following areas should be covered :-

- The findings of the safety patrol

- A review of specific hazards, significant incidents and accidents.

- A review of unsafe acts and unsafe conditions observed and propose remedial measures. Discussion on safety drills held.

Minutes must be taken and circulated to the parties concerned, and one copy displayed on Safety and Health  notice board on site.

The Safety and Health committee must be headed by the Project Manager / Safety Officer. A safety patrol shall be conducted around the job site by the safety officer everyday. The safety patrol shall inspect and check any violations against the staled safety rules.

Particulars attention shall be paid to disclosing hazards or critical working conditions due to a lack of awareness by inexperienced workers. A file of the safety patrol reports shall be kept at the job site during the entire construction period.

# RESPONSIBILITY

# Client's Safety and Health Officer / Engineer

The Client's Safety and Health Officer / Engineer  will cooperate with Contractors Safety and Health Officer  committee to carry out the work safety during the construction period. The responsibility of the Client's safety engineer is to observe that Contractor and all sub-contractors abide by this manual. Any violations against the stated safety rules.

# Contractors Project  Manager

Contractors Project Manager shall be responsible for safety, including fire Prevention, during the construction period. He shall organize and preside over the Contractors committee to ensure that the work is executed safety by all personnel. Also, he shall assign Contractors safety officer.

# Contractors Construction Superintendent

The construction superintendent shall assist the manager and direct the field supervisors for safety. The construction superintendent shall preside over the safety committee in the absence of the manager.

# Contractors Safety and Health Officer

The Contractors Safety and Health Officer shall assist the construction manager and conduct the safety programs under the direction of the construction manager. The Contractors Safety and Health Officer shall check each work plan or procedure for safety before the commencement of work. He shall give us his opinion or concept to the field supervisors.

# Contractors Field / Site Supervisors

The field supervisors shall check and confirm the work procedure regarding safety the commencement of work. They shall supervise the work concerning safety, such as safety programs specified in this manual. If violations or unsafe work conditions are detected, the field supervisors shall immediately advise or instruct the sub-contractor's supervisor or the workers to correct them. Work shall be stopped until violations or unsafe working conditions are corrected.

SECTION 2 : PROTECTIVE SAFETY EQUIPMENT

# GENERAL

Everyone will be provided with the protective equipment so far as is reasonably practicable for their health, safety and welfare.

Every employed person must use the protective equipment provided for his health and safety.

Safety helmets and safety boots are mandatory outside accommodation areas, offices and rest rooms in Restricted Areas.

Protective and safety appliances must be kept clean. Visitors to site must be provided with safety helmets.

# SCAFFOLDING

Scaffolding must be of steel and to be provided for external plaster or their finishes.

It must sit on a firm and leveled base, and be tied back to the structure at every alternative floors.

It has to be covered with safety net to prevent objects from falling out of the building.

# HEAD PROTECTION

On all installation, safety helmets approved to BS 5240 must be worn at all times outside accommodation, offices and rest areas. Safety helmets are for:-

Protecting the head from impact of falling objects or bumping again stationary objects.

Containing the hair and prevent it from becoming entangled with moving machinery.

All safety helmets must be fitted with a chin strap, which will be worn in position during work in elevated positions and in high winds.

Specialized head protection must be used for work. with grist blast machinery and welding.

# FOOT PROTECTION

Only approved safety footwear is to be worn within all Restricted Areas or  work site. Safety footwear is fitted with concealed steel toe-caps, ankle padding and stoles of a non-slip material which should conform to BS 1870 Canvas shoes or sandals must never be worn outside accommodation areas, office and rest areas. Visitors to site may wear robust footwear of a non-safety variety, provided they have low heels and non  spark inducing soles.

# HAND PROTECTION

Industrial glove are available in a wide variety of materials, including Asbestos, P.V.C, Rubber, Polythene, Leather, Nylon etc. to afford protection against heat, electric shock, greases, sprinters, caustics acids and the steadily increasing use of materials with aggressive chemical constituents.

The selection of the right type glove to give maximum protection in any operation is a matter for careful considerations of all the factors involved. The various recommended usage is indicated below :-

Leather or leather-palmed gloves for handing wire ropes.

Cloth gloves for handling pipes.

c)   Rubber gloves for handling acids, caustic soda and soda ash.

d)   Approved rubber gloves for electrical  work. All gloves should comply with BS 1651.

Hand and arms must be thoroughly washed on the removal of any safety gloves, particularly at the end of a working periods and before meals.

# EYE AND FACE PROTECTION

All eye and face protection will meet the requirements of British Standard 2092 and be so marked. Protective goggles, safety spectacles, visors, face shields or screen must be worn to protect the eyes from injury by hydrocarbons, injurious light rays, chemicals, welding arcs, sparks airborne particles and other substances.

# SAFETY BELT

Safety belt must be used whenever  there is danger of falling from high exposed position or into water.

Safety belts will conform to BS 1397.

## SECTION 3: FIRE PREVENTION AND FIRE FIGHTING

# GENERAL

Strict observation of safe working practices and the maintenance of all fire-fighting and other equipment in good working order, together with a high standard of housekeeping, will greatly contribute to reducing the possibility of a fire to a minimum.

# PRECAUTION

Waste bins should not be used as ash trays.

To prevent accidental or spontaneous ignition, oily waster, rags and clothing should be stored in closed metal containers, which should be emptied frequently.

Paints and flammable liquids and materials must be properly stored. The quantity flammable materials, chemicals and acids must be kept to a minimum and chemicals, acids or other substances which react if combined, must be segregated.

Power supplies, including domestic and office supplies, must not be overloaded.

Flammable or dangerous substance must not be permitted to enter drainage or any other water source, unless it is a closed drain system provided for that purpose.

When leaving office, rest rooms and canteens unoccupied, care should be taken to ensure that cigarette ends are extinguished, electrical apparatus is switched off and plugs removed and all doors and windows closed. Compressed gas cylinders containing propane, butane, oxygen and acetylene must be stored in a safe area and suitable segregated. Extreme care must be taken to ensure that no ill or grease contaminates oxygen cylinder valves.

# THE PHENOMENON OF FIRE

Fire has three essential ingredients: fuel, oxygen and heat.

Fire will occur in the presence of these three elements. If one or more of elements of the fire will be extinguished. This can be done by:-

Starvation i.e. removal of combustible material. Smothering i.e. excluding of oxygen (or air)

Cooling i.e. lowering the temperature of the burning material below its ignition temperature.

One way of smothering a fire is to drive away the oxygen contained in the air from the vicinity of the fuel and to replace it with carbon dioxide or other gases which will not support combustion.

Another technique is to apply a dry chemical powder in the form of two or all of these methods, although anyone will subdue a fire.

# PORTABLE FIRE EXTINGUISHERS

The quantity and type of extinguisher installed must be appropriate to fires which may occur.

They must be located so that they are not exposed to the danger of damage from heat, corrosion, objects or materials and are quickly accessible.

They must be inspected monthly and kept in perfect working condition.

No one type of extinguishers will deal with all types of fire, but in general the following guidelines should be followed:-

a) BC Purple K base type of Dry Chemical Fire Extinguisher  for process areas.

b) ABC Multipurpose Dry Chemical Fire Extinguisher  for utility and accomodation areas

c) BCF ( Bromochlorodiffiorodiffiouromethane) Fire Extinguisher  for areas with predominate electrical risk. All onshore fire extinguishers are subjected to Jabatan Bomba yearly certification.

# FIRE - FIGHTING

It is essential that personnel must :-

a) Know the location of all emergency equipment.

b) Know when and how to use all emergency equipment.

c) Maintain all equipment in a clean and opera table condition.

Prompt, confident action can prevent the rapid development of the fire. The provision, sitting and storage of all firefighting appliances has been carefully planned with regard to the requirements of a particular area. A thorough knowledge of the usage and operation of all fire fighting appliance and their storage is essential, if a good organisation is to be effective in controlling and extinguishing any fire which may occur.

It  is  essential  that  all  portable  appliances  are  kept  fully  charge  and  are tested at regular intervals. The function and serviceability of these appliance can be ensured  only by a regular test and training.

SECTION 4 : MEDICAL FACILITY AND FIRST AID

FIRST AID

Definition of first aid is an aproved method of providing immediate temporary assistance to a casualty of an are to preserve life, prevent worsening of the patient's condition and to stimulate recovery.

It accident of sudden illness until treatment can be administered by a medic/qulified medical practitioner. The objectives is required that Contractors officer should be familiar of basic first aid procedures and be able to act quickly in an emergency to preserve life by artificial resusciation, heat stimulation and control of bleeding.

# PRECAUTION AND LIMITATION

In emergencies, there is no subtitute for good common sense and elimination of excitement. When in Doubt regarding the validity of a procedure,attempt should be made to obtain competent assistance. Errors in tecnique of first aid procedures,as well as erroneous interpretations of the symptoms,may carry grave or fatal consequences.lt is particularly important that good judgement should be exercised.

# GENERAL FIRST AID PROCEDURES

Keep the injured persons (s) lying in a comfortable position, with the heal level with the body,until the nature of the injuiry can be vertified.

Qiukly ascertain whether there are 4 any evidence of:-

a) Shock

b) Respiratory prolems

c) Severe bleeding

d) Serious burns

Note whether there are other lessers wounds, fractures or other injuries.lf the unconscicous victims was involved in a violent accident, suspected a head and/or back injury.

Keep the injured persons warm.

Move the person only to protect them for further injury or when adequate professional help is present. Give nonthing by mouth to unconscious individuals. Keep onlookers away from the injured.

Condition at the accident may hamper the worked in giving goods\first aid: Keep Calm.

### BLEEDING

Uncontrolled arterial bleeding can cause death within minutes.Severe bleeding should be stopped by applying direct pressure on the artery. Elevation of a bleeding limb will slow blood loss.

Procedure:-

1. Lay the victim down, if not already so, and keep him still.

2. Place a pad made ideally using sterile gauze from the first aid kit or otherwise from a clean piece of cloth,handkerchife,etc. Direct over the wound and press fimrly to stop he flow of blood. If you do not have a pad of any sort,close the wound with your hands or fingers and prss hard to stop the flow of blood.

3. If possible, tie the pad firmly in place with a strip of cloth ,handkerchief or the like.

4. If you sure that 0 bones are broken,raise the damage part above the rest of the body.

# BURNS

Burns be caused by chemicals, dry heat, wet heat (scalds) and harmful rays. All burns can be complicated by shock, and prime objectives in treatment are to prevent or control shock, pain and infection.

Chemical bums may arise from skin contact or ingestion. For skin contact flushing with copious quantity of water and removal of affected clothing or by rolling the victim in bIanket or coat in adequate prior to specialist attention.

Thermal burns will become infected quickly and medical help must be summoned relive pain and stop the burn from damaging tissue further, the use of cold, clean water should be considered. Do not disturb any blisters formed and exclude air by covering the area loosely with clean material. Removal of any clothing or debris must only be out by trained medical personnel.

# SHOCK

Shock is the failure of many body functions due to inadequate circulation. Severe shock is a serious condition and may result in death. Shock normally accompanies severe injury or illness and must be quickly treated to prevent worsening of the condition.

Possible causes of shock are severe or extensive injuries, pain blood loss, electric Shock and heart attack.

Symptoms of shock are cool, clammy skin, pale complexion, a weak or rapid pulse, nausea, faintness, perspiration, shallow or arrested breathing, disturbed mental condition and unconsciousness. To treat a person suffering from shock, whose breathing is arrested, apply artificial respiration. Try to correct the cause, for example, control any bleeding. Lay the patient's head to one side in case vomiting occurs and keep the airways clear. Keep patient warm, but do not overheat as this draws blood from vital organs to the skin, and do not give patient anything to drink; as surgery may be necessary.

# FRACTURES

The patient should not be moved unless absolutely necessary until fractured limbs are immobilising, in order to avoid fracture from further complication. When faster tie the limb both above and below the site of the fracture.

Fractures are divided into two types :-

Simple fracture -broken bone but no open wound

Compound fracture - broken bone which may have puncture the skin or is associated with nearby wound procedures :-

Never move an injured person unless the injured is well splinted, or it is necessary to save the victim from further injury.

If the break is in a limb, place it in a normal position as possible, without causing excessive pain. Have someone support the limb above and below the fracture to immobilise the area.

In the case of compound fracture, control the bleeding and apply a sterile dressing before splinting.

Apply well padded splint made of any rigid material, such as wood. Make it long enough to extend beyond the joints and below the injury.

Fasten the splints with bandages or wide strips of cloth in three or more places so that the joints and points of break are immobilised.

If there is suspicion of broken spine, take EXTREME care and keep the victim on his back with his head in line with his spine and well braced on each side. Do not attempt to apply splints to neck or spine fractures.

If there is suspicion of a fractured skull, take EXTREME care and keep the victim quite. If he is unconscious, remove any foreign matter from his mouth and turn his 'head to one side in order that any secretion may drain from his mouth.

# EYE INJURIES

All eye require medical attention. Adverse developments can sometimes seriously impair vision. Avoid rubbing or the use of instruments in the removal of foreign bodies.

### Foreign Bodies in the Eye

1. The person administering first aid must wash his hands first and foremost.

2. A loose foreign body in the eye which can be seen may be removed with a moist sterile cotton tip.

3. Apply an eye pad or loose dressing and refer to physician or medic

##### Wounds to the Eyeball

Do not attempt any first aid in the case of wound to the eyeball. Assistance may be :-

1. Apply a sterile eye pad and loose dressing.

2. Keep the victim lying flat.

3. Treat for shock

4. Refer to a physician as immediately as possible.

### Burns of the Eyes

In the case of chemical burns the procedures is to :-

1. Immediately flush with water for a minimum of 20 minutes and longer if pain continues.

2. Keep eyelids open during flushing.

3. Apply a sterile pad and dressing.

4. Refer the patient to a physician, preferably on ophthalmologist.

Do not apply boric acid or other chemicals.

# HEAT STROKE

This result from prolonged exposure to the sun. The victim will be a skin and strong rapid pulse. He may be unconscious.

1. Call the medic or doctor.

2. Sponge the victim's body with water to reduce fever. If the victim is fully conscious, give him salt water (one teaspoon of salt to one litre of water).Altenatively give him salt tablets (dosage stated on the container).

3. Do not give alcohol in any form

When working in direct sunlight, wear protective clothing or limit working hours to mini mise the possibility of heat stroke. HEAT STROKE CAN BE FATAL.

# GENERAL HYGIENE

Regular hygiene inspections will be carried out by the Medic and the supervisors and superintendents to ensure that all safety and hygiene standards are being observed. Upon conclusion of a hygiene inspection, any unsatisfactory condition highlighted by the inspection team must be remedied in accordance with laid down procedures and management directives. Particular important subjects to be covered are:-

Personal hygiene, especially skin care and prevention of dermatitis.

# Kitchen and food handling

Refuse disposal

Personal Hygiene

Personal cleanlines should be encouraged .Certain operations and the handling of some subtances may given rise to dermatoid and other skin diseases. The skin must be kept clean and in a healthy condition and appropriate protective clothing must be worn. Washing of hands before taking food is essential in all cases, particularity so when works involves the handling of leaded products, chemicals, etc.

Care must be taken to ensure that clothing is not heavily soiled. Clothing contaminated by dangerous substances should be remove as quickly as possible, preferably under a shower. The clothing should then be laundered under strict supervision or if this is not possible, it should be destroyed. All working clothes Should be 'racially washed. Regulation to govern the use and cleaning of protective clothing under controlled conditions may be necessary.

Kitchen and Food handling.

The standard of hygiene for food preparation rooms and food stores shall conform to the standards laid down in statutory regulations. The walls, floors, ceilings, woodwork and all other parts of the food rooms must be kept in good order and repair to enable them to be effectively cleaned, and to prevent, as far as possible any risk of infection by rats, mice or insects. Regular inspections shall be carried out by the relevant supervisors. No refuse, solid or liquid, will be allowed to accumulate in any kitchen or food storage room.

All personnel seeking employment within the food services section must first undertake a food handler's health examination. No person found to be unfit by this examination shall be hired for employment. Personnel will be required to pass any. Subsequent examination. Accurate medical records will be kept to provide, on a regular basis, details of those individuals whose medical examination are due.

##### Any personnel engaged in handling food must;-

1. Maintain high standard of personal cleanliness. They will thoroughly wash their hand before starting work and during work as often as is necessary to keep them clean. This will include after smoking, eating, drinking and going to the toilet.

2. Keep their clothing as complete as possible, clean uniforms must be worm by personnel on duty. The uniforms notes hat and foot wear.

3. Refrain from spitting.

4. Smoking be refined in a food storage, preparation or service area

5. Report to their supervisor immediately when they become aware that they are suffering from diarrhea or any communicable disease likely to cause food poisoning. The supervisor will, notify the medic/superintendent, who will then notify the appropriate health authorities.

The use of equipment and utensil in food areas which meet or improve existing sanitation standards is recommended. The standards require easy cleaning, durability and sound construction so as to minimise the chances of for food remain on the equipment. It is essential that all utensils are washed and sanitised in accordance with approved procedures.

All food should be stored in approved covered containers in ,accordance with proper storage procedures. Coolers and freezers should be cleaned according to laid down cleaning schedules and during periods when the least amount of food is present. The proper handling of food to high standard of sanitation. Food should be prepared with the least possible hand contact, with suitable utensils and on surfaces that have been cleaned, rinsed and sanitised.

SECTION 5: SITE TRANSPORTATION

MOTOR VEHICLES

All company motor vehicles must be driven by an authorised company driver holding a valid driving license.

All drivers must be familiar with and abide by plants or sites, national, slate and traffic regulation.

Equipment and all company motor vehicles must conform to the Road Transport Department requirements.

Safety belts are installed in all vehicles. All drivers of company vehicles MUST use such equipment and request passengers to use them.

Getting on or of a vehicles while its engine is running must not allowed. If a servicing unit equipped with an engine is used to fuel the vehicles, the engine of the servicing unit should also be shut off if its power is not needed to deliver the fuel.

Smoking must not be allowed near the vehicles while it is being refueled.

Flammable liquids are not to be caned in trucks or luggage compartments of vehicles equipped with two-way radios. However sample bombs handled by gas tester and other authorised persons may be carried in such vehicles provided the valves are plugged. to prevent leakage.

To prevent the accumulation of poisonous carbon monoxide fumes in an enclosed space, doors must be opened or exhaust fans must be utilized for ventilation whenever a truck or passenger car motor is running. Remember that these deadly fumes are colorless and invisible.

Motor vehicle shall not be driven within areas such as around tank farms, burning pit and low points where combustible gas may have accumulated.

### SECTION 6: WELDING AND FLAME CUTTING

# GENERAL

Gas and electric welding, cutting, brazing and soldering are all safe operations when carried out by skilled persons trained in this work, and when work is carried out under proper conditions. Equipment should be well maintained and good ventilation and lighting together with a tidy work area are important factors in safe working.

Barrels, drums, etc, should not be used to support work in view of the explosion risk in the event of accidental flame impingement on such receptacles. This is apart from instability of the drums, which are not designed for this purpose.

In all welding and burning operations, consideration must be given to the space between double plates, where flammable material may be found.

The igniting of oxy-acetylene propane torches must only be made by using the 'Gas Lighters' specifically supplied for that purpose. On no account must disposable butane lighters be used as they are considered dangerous.

# PROTECTIVE CLOTHING

Care must be taken to ensure that clothing is free of oil and grease and that pocket sand cuffs are not open and ready to receive sparks or hot slag.

Leather gloves, gauntlets. jackets and aprons should be worn, as necessary, to protect the operator from sparks and the radiation of the arc. Leather gloves or gauntlets give little insulation against electrical hazards.

Eye protection is of utmost importance, and all welding, cutting chipping and grinding must be carried out using goggles, screen, helmets and shield, as required. and in accordance with B.S. Specification.

Other person, not immediately engaged in work, but who may be affected by hazards associated with it, must be adequately protected. This may require the provision of ant-flash eye protection or protective screens.

A common cause of eye injury is the chipping away of slag covering the weld when coated electrodes are used. In this circumstances a helmet with double other (the outer) used, one screen consists of a clear safety glass window while the screen can be simply adjusted to bring either one or both screens into use. Alternatively clear glass safety goggles may be worn in conjunction with the use of a hand screen.

# TOXIC FUMES AND VENTILATION

Welding cutting and brazing operations can produce mixtures of gases, fumes and smokes the compositional which depends on welding temperatures, arc intensity, base metal, electrode type, etc. Strict ventilation precautions including the use of suitable breathing apparatus must be taken to prevent inhalation of fumes from welding and cutting processes in which the component metals, electrodes or luxes contain lead, cadmium or other toxic substances.

With the introduction of inert gas shield welding, and as a result of the use of higher currents ozone may be formed in amount that could be dangerous. This method of welding must only be used in well ventilated areas together with suitable breathing apparatus.

The welding of greasy material can produce fumes, and accordingly, such materials should be degrease before welding. Parts which have been degreased should be dry before welding.

# PREVENTION OF FIRE AND EXPLOSIONS

Persons carrying out or assisting in any welding or flame cutting operations must be alert at all times to the dangers of fire or explosions.

Fire watchers should be appointed for each welding or cutting operation and fordable fire extinguishers kept in close proximity to the work site.

All cylinders, hoses and fittings should be regularly checked for leaks. This is especially true if the equipment to be used in a confined space, for example, a vessel or tank. No cylinders must be taken inside and enclosed space

Welders must never coil hoses around their body when welding.

The blowpipe from any oxygen, propane or any acetylene, when alight, must not be on the cylinder or regulator.

Ensure that sparks, slag, hot metal and the welding arc or flame cannot make combustible materials, work areas must be screened to prevents etc. From flying outside the immediate welding are. Screening material should be fire proof and the area may be required to be kept well.

When working at a height, precautions must be taken to prevent welding rods and stubs from falling, screening will be also be required, particular attention being paid to the area immediately below the welding area.

Ensure that equipment is turned off at source, e.g. gas cylinders or welding set, whenever it is left unattended.

# GAS CYLINDERS USED FOR GAS WELDING AND BURNING

Cylinders should be stored in a specially designated area and protected from the direct rays of the sun and inclement weather. The area should be free of flammable materials and isolated from any heat source. The cylinders shall be stored in such a manner that they can be readily removed in the event of a fire.

If oxygen and acetylene cylinders have to be stored in the same room they must be kept well apart. Preferably, they should be stored separately.

Full and empty cylinders must be clear distinguish shed and stored apart. Empty cylinders should be marked either 'EMPTY' OR 'MT' .

Propane and acetylene cylinders must always be store vertically and secured Oxygen cylinders may stacked horizontally pyramid fashion but not more than four hit i.e. 10 cylinders. The largest cylinder must be at the bottom of the stack, and the stack securely wedged.

Oxygen cylinders hoses and fittings must not be stored or used where they may be contaminated by oil or grease. This includes handling with oily hands, gloves or  rags. Oil and greases are spontaneously combustible in the presence of oxygen.The valves of cylinders must always be kept protected. The use of valve caps is recommended here available.

Cylinders must only be transported improper cases or trolleys, secured to prevent movement, and lever subjected to excessive shock or rough use.

Cylinders must never be used for work supports, rollers, or jacks.

Cylinder valves should be checked frequently for leaks with soapy water to eliminate the risks of leaks and possible explosion.

Cylinders must not be allowed to come into contact with electrical equipment or be sited near any sparks, flames or hot slag during welding and cutting.

# ELECTRICAL ARC WELDING

All welding assets, cables, connectors and other equipment must be maintained in good condition. Welding machine should only be connected to a supply that can be quickly isolated mine event of  emergency.

Where stationary transformers or generator sets are being used, a suitable switch must be mounted adjacent to the equipment in order that it may be isolated from the supply main. Engine driven machines must be approved for the area in which they are sited and never refueled whilst in operation.

All machines must be properly earthed. Pipelines or electrical conducts must be for earthling purpose. Only electrode holders with handles of tough insulating material must be used.

# WELDING LEADS

A welding return lead must be provide from the welding machine to the workplace where it must be firmly secured by bolting or clamping.

All weldings leads and returns shall consist of flexible tough rubber covered cable connect where necessary with approved cable couplers. All lead shall be proper terminated and lead cable couplers and termination shall be of adequate current capacity.

Portable transformers, or generator set, having trailing leads should be provided with interlocked fused switch sockets and plugs, protecting both trailing cable and equipment.

To avoid long lengths of power cables the machine must be sited as close as possible to the power supply points.

Where more than one operator is working from the same welding machine the polarity of each welding lead and welding return shall be the same.

When two or more leads are being used in a confined area the corresponding leads must be paired and, as far as possible, the crossing of pairs with one another or with any single lead should be avoided.

In instances where welding leads trail near tanks or sumps containing hydrocarbons, additional lengths of cable must be added to the circuit to circumvent the danger area.

Cable must be in good condition with no taped joints or repairs. Inspection of all trailing leads, from lachine to work piece, must be carried out regularly by the welding supervisor.

# SECTION 7 : CRANE OPERATIONS AND HOISTING

# GENERAL

Cranes are designed to raise/lower load safety at their designed capacities supervisors must be familiar with the characteristics and capacities of the crane being used and crane operators must be properly trained to operate the crane safety. All lifting appliances and lifting gears must be tested and examined to the relevant legislation, where applicable.

Crane will only be operated by persons who have been trained. No personnel are allowed to ride on the hook or load. Crane hoisting mechanisms must never be used for any purpose other than lifting.

Crane jibs should never be tied down to pipelines or equipment supports.

Crane activities will stop in the event of any emergency or where potentially dangerous situation is expected.

# MECHANICAL CONDITION

Cranes and lifting equipment must be in good mechanical condition and assembled in accordance with the manufacturer's instructions. Deviations from those instruction shall not be permitted.

All control levers switches and buttons shall be clearly marked to indicate their function.

The driving and operating cabs shall be kept free of all rubbish and unnecessary material .All broken glass should be replaced.

# CRANE OPERATOR

Crane operators must be competent .to operate the particular type of crane. He must also be able to :-

a) Know and comply with standard lifting hand signals.

b) Read and understand operating instructions.

# SLINGER/RIGGER

The Slinger/Rigger is responsible of attaching the load to the crane and for giving the correct hand  signals to the crane operator. Standard signals must be used. Normally, all signals must be given by the Slinger/Rigger but the Crane Operator must obey an emergency stop signal given by anyone.

# OPERATING PRECAUTIONS

The safe working load, as specified in the manufacturers instructions shall not be exceeded at any time. The load shall be taken smoothly. Snatching can cause boom or rope failure Where the load is close to the operating maximum, the load shall be raised only a few inches and the crane checked for stability. Loads shall not be dragged or pulled sideways. The places side stress on the boom and overloads the crane.

Swinging shall be done slowly. Swinging too fast can throw the load out of radius and cause overloading and the risk of striking adjacent objects.

When a crane has a swinging motion, a clear space shall be maintained between the crane body, the counterweight and any moving parts of the crane and fixed objects nearby to prevent persons being trapped and crushed when the cranes wings.

# OPERATIONS OF CRANES

The crane operator shall examine the following before the crane:-

- crane foundation

- drum dogs, brakes, boom, guard and pins.

- load hooks, wire line cables, fittings and fastening and sling.

The slinger/Rigger Crane Operator shall jointly check the load chart, cornfirm the boom length with chart, establish the maximum load in relation to the maximum operating radius or the corresponding minimum boom angle.

Stops should be provided to the boom to prevent "overtopping".

When shutting down, the crane and boom must be properly secured, the supervisor in charge be informed of any defects on the machinery.

Wherever possible, safety belts or latches shall be used on crane hooks during lifting operations.

Maintenance inspection records shall include the inspection date and person out the inspection.

# LOAD INDICATORS FOR CRANES

All cranes shall be fitted with an appropriate safe working load indicator and a loading chart shall be placed inside the crane cab where it can be seen easily.

The indicator shall be of such a type that the operator can readily ascertain the safe working load of the crane at the particular boom length and radius at which he is working

It shall be the responsibility of the supervisor-in-charge to make arrangements that the weight of any load to be lifted is indicated to the slinger and operator.

Where automatic load indicators are fitted, they shall be properly calibrated and the appropriate cams fitted.

# LIFTING FEAR AND TACKLE

All chain, ropes and lifting tackle must be of adequate strength constructed of sound material and be free of any obvious defect.

Remove from service all defective rigging equipment.

Do not tie knot in chains, rope slings or wires to shorten them.

Do not place bolts or other material between links of chain to shorten or splice it.

Protect slings from damage from sharp edges by wrapping the edges with a softer packing material. Avoid sharp bends in slings and protects slings from abrasion. Set down loads on proper blocks, never directly on a sling.

AIways screw in eye bolts up to the collar, otherwise a sideways pull will  overstress the shank, if necessary, use a slings.

SIGNALLING.

Use proper signals with accepted code only.

Only one person must be in charge of giving the signal during the lifting operation . Signals should only be given when the operator can easily see them.

Any signal that is not properly understood should be treated as a stop signal.

No signal for the movement be given until the signalman is satisfied himself that no person in the area for which he is responsible will be endangered by the movement.

Signalman should have no other duties when signalling.

Signalman should be prepared to given the stop signal at any moment while signalling and be fully alert at all times.

During night works, it would be better to gives audible signals and man should wear clothing of height visibility

# RADIO COMMANDS

Every instruction must be followed by the word 'over' to signify the end to the command or instruction. Prior to commanding the tower crane one must identify himself and advise his actual location. Strictly no chatter is to be permitted when using the walkie-talkies. Speak clearly and slowly.

Use concise instructions so as not to make the command talking time too long.

When the operator is unable to see the lifting or landing zone, give a metal picture of the surrounding  area and state precautions to be taken and if any obstruction are in the way.

Always inform the operator on:-

- Type of load to be carried.

- Approximate weight or quantity.

- Where to be lifted.

- Where to land the load.

- Any obstructions or additonal precautions

- The speed required.

CORRECT SLING PRACTICE.

Used sling which have a known safe-working capacity whether individually or in a group.

All lifting gear should be maintained and inspected in accordance with the manufacturers instructions.

Only gears in good condition should be used during hoisting operations.

Never use damaged slings unless they have been repaired by specialist

Never overload the lifting equipment and gear.

The sling must be positioned correctly by avoiding placing around sharp edges and not to let it slide over comers or along edges.

Never drag goods with the sling.

Position the sling correctly to ensure easy removal after use by placing bearers on the floor for the load to sit on.

Take extra care when lifting heavy cargo with more than one sling because the total weight may not be evenly distributed.

Sling should be examined at intervals of not more than two weeks when used for general lifting. When moving long objects always use guide ropes to prevent swaying or swinging.

Always check to ensure packing case is in good condition and never try to lift worn or damaged packing case.

Use the correct accessories for the right job like lifting beams, clamps, hooks, shackles, eyebolts, etc.

Never operate in conditions of strong wind and poor visibility.

Note that all sling material looses strength with time and they should be down graded in permitted lifting capacity with time.

Have a list of the weights of the common items, which is lifted daily to avoid determining the load by trial and  error method.

POINT TO REMEMBER DURING LIFTING OPERATIONS.

See to ensure the load is free before lifting and commence lifting slowly.

Always try a load to see that it is securly slung first and never move a load unless it is safe.

Never allow a load to be carried over the heads of other people and if necessary give warning for them to keep clear.

Watch out for obstructions and keep the load well clear when moving a load down. Never carry unused wire ropes, slings or chain on the same hook carrying a load

Do not leave the load suspended over a long time if it is necessary to make a temporary stop while lifting a load.

Either remove the sling or place the sling hooks in the ring before giving the finish signal.

If the hook is in constant use ensure the sting and the crane hook is clear and will not catch on anything.

A flasher or blinking light should be provided on the moving load during the hours of darkness.

BAHAGIAN KESELAMATAN

DAN KESIHATAN  PERKERJAAN

BAHAGIAN PENGURUSAN FASILITI

UNIVERSITI TEKNOLOGI MARA

SISTEM PENALTI  DALAM PERLAKSANAAN TATACARA ETIKA KERJA, GARIS PANDUAN DAN PERATURAN KESELAMATAN DAN KESIHATANPEKERJAAN

( KKP ) DI TAPAK BINAAN UiTM.

| NO | KESALAHAN | PENALTI |
| --- | --- | --- |
| 1.0 2.0 3.0 4.0 4.1 4.2 4.3 4.4 4.5 4.6 5.0 6.0 7.0 8.0 8.1 8.2 8.3 8.4 9.0 9.1 9.2 9.3 9.4 10. 11. 12 13 14 | Gagal menghadiri krusus keselamatan dan kesihatan pekerjaan dari bahagian Keselamatan dan kesihatan Pekerjaan (BKKP-UiTM) sebelum memulakan sebarang apa-apa kerja di UiTM. Gagal mematuhi peraturan berkaitan pemakaian pakaian perlindungan ketika berkerja atau berada di tapak binaan seperti topi dan kasut perlindungan PPE.( Personal protection equipment ). Merokok di dalam kawasan tapak binaan atau kawasan / zon UiTM. Melaksanakan kerja-kerja yang berkaitan haba tinggi( hot works) tanpa perkiraan : ( Hot permit) yang sah laku tempohnya. Alat cegah kebakaran yang tersedia ada di lokasi tapak (stanby fire extanguisher). Pemasangan flashback arrestor pada acetylene cylinders. Perlindungan yang sempurna di kawasan persekitaran operasi jentera. Papan tanda peringatan/amaran yang sempurna dan cukup. Silinder gas (welding & cutting) berada dalam keadaan tegak di dalam troli. Menggunakan atau berkerja di atas perancah yang tidak sempurna dan berbahaya didalam tapak binaan UiTM. Tidak menggunakan tangga sempurna dan sesuai semasa bekerja pada aras yang tinggi di tapak binaan /operasi penyelengaraan. Tidak menggunakan penghadang sempadan ( barricade) yang sempurna : Di perimeter tapak binaan. Di kawasan kerja tebus guna tanah. Di lantai dan dinding yang terbuka. Di bahagian bawah tapak dimana kerja-kerja yang melibatkan aras tinggi. Penggunaan alat-alatan elektrikal: Terdapat kerosakan pada kabel dan bahagian- bahagiannya. ( damage part ). Penggunaan kabel yang sambungannya tidak selamat ( unsafe connection ). Dengan kerosakan atau tiada penutup atas plug /soket elektrik yang digunakan. Tiada pendawaian bumi ( earth circuit breaker ELCB dan pendawaian bumi bagi generator yang tidak sempurna. Mengunakan jentera atau mesin pembinaan seperti mobile crane, lifting crane, skylift, scissors lift, air compressors, portable generator,welding generator,chain block,lever block,wire rope,lifting belt dan shackel tanpa tag pemeriksaan keselamatan. Menggunakan operator atau pekerja yang tidak berkelayakan/kompeten dalam operasi jentera di tapak binaan. Bekerja dalam kawasan/ruang tertutup seperti tank,vessel,mainhole atau sewer tanpa kebenaran/kelulusan UiTM. Tidak mematuhi mana-mana peraturan dan syarat dalam tatacara etika kerja,garis panduan dan peraturan keselamatan dan kesihatan pekerjaan di dalam operasi tapak binaan Pejabat Pengurusan Fasiliti atau Pejabat Pembangunan yang berkaitan Keselamatan,kesihatan dan hal-hal persekitaran. Aktiviti pergaduhan sesama pengurusan kontraktor atau pekerja dan melakukan jenayah di kawasan tapak binaan UiTM Disyaki menggunakan dadah atau alkohol semasa bekerja atau ketika berada di dalam tapak binaan UiTM. | RM200.00 RM200.00 RM100.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 RM200.00 Dibuang dari Tapak Bina dan seluruh kawasan UiTM Dibuang dari Tapak Bina dan seluruh kawasan UiTM |